yomelelani damane

damaneyomelelani@gmail.com

51942437

COMPUTER SCIENCE

Ditiro-Financial-Services-Taung

MILESTONE TWO

This project involved the design and implementation of a segmented enterprise network using Cisco Packet Tracer. The network was designed to support multiple departments while separating departmental traffic into dedicated VLANs.

A Cisco Catalyst 3560 multilayer switch was used as the core Layer 3 device to provide inter-VLAN routing, while three Cisco Catalyst 2960 switches were used as access switches. A router was connected to the core switch through a routed point-to-point link.

Network security was implemented using Access Control Lists (ACLs), including Guest network isolation and controlled access to the server network. Secure SSH management was also configured for the access switches, and unused switch ports were hardened by placing them into an unused VLAN and administratively shutting them down.

Project Objectives

The objectives of the project were to:

- Design a structured enterprise network topology.

- Segment departments using VLANs.

- Configure Layer 3 inter-VLAN routing.

- Configure 802.1Q trunk links between switches.

- Implement network security using ACLs.

- Isolate the Guest network from internal networks.

- Restrict access to the Server VLAN.

- Configure a dedicated Management VLAN.

- Configure secure SSH access to network switches.

- Harden unused switch ports.

- Configure static routing between the core switch and router.

- Test and verify end-to-end network connectivity.

- Troubleshoot configuration and connectivity problems encountered during implementation.

Network Requirements

The network was designed to support the following departments and infrastructure services:

- Administration

- Finance

- Human Resources

- IT

- Guest

- Server

- Network Management

Each department was assigned a separate VLAN and subnet. The network was designed so that authorised departments could access the server while Guest users were prevented from accessing internal networks.

Network devices were assigned management addresses on a dedicated Management VLAN and configured for secure SSH administration.

Network Topology

The topology consists of one Cisco Catalyst 3560 multilayer core switch, three Cisco Catalyst 2960 access switches, one router, departmental end devices, Guest devices, and a dedicated server.

The Catalyst 3560 provides Layer 3 routing between VLANs and connects to the router using a routed point-to-point link. The three access switches provide connectivity to departmental end devices and connect to the core through 802.1Q trunk links.

Device Inventory

Core Network

- Cisco Catalyst 3560 Layer 3 Core Switch

- Cisco Router

Access Layer

- Cisco Catalyst 2960 SW1

- Cisco Catalyst 2960 SW2

- Cisco Catalyst 2960 SW3

End Devices

- 2 Administration PCs

- 2 Finance PCs

- 2 HR PCs

- 2 IT PCs

- 2 Guest PCs

- 1 Server

VLAN Design

VLAN 10 — Administration

- VLAN ID: 10

- VLAN Name: ADMINISTRATION

- Network: 192.168.15.0/27

- Gateway: 192.168.15.1

- PC1: 192.168.15.2

- PC2: 192.168.15.3

VLAN 20 — Finance

- VLAN ID: 20

- VLAN Name: FINANCE

- Network: 192.168.15.32/27

- Gateway: 192.168.15.33

- PC1: 192.168.15.34

- PC2: 192.168.15.35

VLAN 30 — HR

- VLAN ID: 30

- VLAN Name: HR

- Network: 192.168.15.64/27

- Gateway: 192.168.15.65

- PC1: 192.168.15.66

- PC2: 192.168.15.67

VLAN 40 — IT

- VLAN ID: 40

- VLAN Name: IT

- Network: 192.168.15.96/27

- Gateway: 192.168.15.97

- PC1: 192.168.15.98

- PC2: 192.168.15.99

VLAN 50 — Guest

- VLAN ID: 50

- VLAN Name: GUEST

- Network: 192.168.15.128/27

- Gateway: 192.168.15.129

- PC1: 192.168.15.130

- PC2: 192.168.15.131

VLAN 60 — Server

- VLAN ID: 60

- VLAN Name: SERVER

- Network: 192.168.15.160/27

- Gateway: 192.168.15.161

- Server: 192.168.15.162

VLAN 70 — Management

- VLAN ID: 70

- VLAN Name: MANAGEMENT

- Network: 192.168.15.192/27

- Gateway: 192.168.15.193

- Core SVI: 192.168.15.193

- SW1: 192.168.15.194

- SW2: 192.168.15.195

- SW3: 192.168.15.196

Router-to-Core Transit Network

A dedicated /30 transit network was used between the multilayer core switch and the router.

- Network: 192.168.15.224/30

- Core 3560 Fa0/4: 192.168.15.225

- Router Gi0/0/0: 192.168.15.226

- Broadcast: 192.168.15.227

Fa0/4 on the Catalyst 3560 was converted into a routed interface using no switchport. The router interface was assigned the corresponding point-to-point address. Connectivity between the two devices was verified using ICMP ping tests.

Physical and Logical Port Assignments

SW1

- Fa0/1 → Administration PC1 → VLAN 10

- Fa0/2 → Administration PC2 → VLAN 10

- Fa0/3 → Finance PC1 → VLAN 20

- Fa0/4 → Finance PC2 → VLAN 20

- Fa0/5 → HR PC1 → VLAN 30

- Fa0/6 → HR PC2 → VLAN 30

- Fa0/7 → Core uplink → Trunk

SW2

- Fa0/1 → Core uplink → Trunk

- Fa0/2 → IT PC1 → VLAN 40

- Fa0/3 → IT PC2 → VLAN 40

- Fa0/4 → Guest PC1 → VLAN 50

- Fa0/5 → Guest PC2 → VLAN 50

SW3

- Fa0/1 → Core uplink → Trunk

- Fa0/2 → Server → VLAN 60

- Fa0/3 → IT device → VLAN 40

- Fa0/4 → Guest PC1 → VLAN 50

- Fa0/5 → Guest PC2 → VLAN 50

VLAN Configuration

Figure 2: VLAN Configuration

The show vlan brief command was used to verify VLAN creation and access-port membership.

Core Switch Configuration

ip routing enabled Layer 3 forwarding on the Catalyst 3560 and allowed the switch to route between VLAN interfaces.

SVI Configuration

interface Vlan10

ip address 192.168.15.1 255.255.255.224

interface Vlan20

ip address 192.168.15.33 255.255.255.224

interface Vlan30

ip address 192.168.15.65 255.255.255.224

interface Vlan40

ip address 192.168.15.97 255.255.255.224

interface Vlan50

ip address 192.168.15.129 255.255.255.224

interface Vlan60

ip address 192.168.15.161 255.255.255.224

interface Vlan70

ip address 192.168.15.193 255.255.255.224

Trunking and Spanning Tree

802.1Q trunking was configured between the Catalyst 3560 core switch and the three Catalyst 2960 access switches. VLANs 10, 20, 30, 40, 50, 60 and 70 were permitted across the trunk links.

VLAN 1 was used as the native VLAN on the trunk links. Native VLAN consistency was verified on both ends of the switch-to-switch connections.

Spanning Tree Protocol using PVST was used to prevent Layer 2 loops in the redundant switching topology.

These following commands were used:

show interfaces trunk

show spanning-tree

Inter-VLAN Routing

Inter-VLAN routing was provided by the Catalyst 3560 using Switch Virtual Interfaces. Each VLAN was assigned a gateway address, allowing devices in different VLANs to communicate when permitted by the configured security policies.

Connectivity to the VLAN gateways was successfully tested from the departmental PCs.

- Administration → 192.168.15.1

- Finance → 192.168.15.33

- HR → 192.168.15.65

- IT → 192.168.15.97

- Guest → 192.168.15.129

Routing Configuration

The Catalyst 3560 was configured with a default route pointing toward the router.

ip route 0.0.0.0 0.0.0.0 192.168.15.226

The router was configured with static routes for all internal VLAN networks through 192.168.15.225.

routes

192.168.15.0/27

192.168.15.32/27

192.168.15.64/27

192.168.15.96/27

192.168.15.128/27

192.168.15.160/27

192.168.15.192/27

Security Architecture

Network security was implemented using VLAN segmentation and extended Access Control Lists.

The main security controls were:

- Guest network isolation

- Server access control

- Dedicated Management VLAN

- SSH for secure remote administration

- Unused-port shutdown

- Unused-port VLAN assignment

These controls were designed to reduce unnecessary network exposure and limit communication between network segments.

Guest Network Isolation

Guest devices were placed in VLAN 50 and isolated from the internal 192.168.15.0/24 network.

ACL:

ip access-list extended GUEST_RESTRICTION

deny ip 192.168.15.128 0.0.0.31 192.168.15.0 0.0.0.255

permit ip any any

Applied to VLAN 50:

interface Vlan50

ip access-group GUEST_RESTRICTION in

The first ACL statement denies IP traffic sourced from the Guest subnet to the internal 192.168.15.0/24 network. The second statement permits other IP traffic.

This command was used:

show ip access-lists GUEST_RESTRICTION

Server Protection

The server is located in VLAN 60 at 192.168.15.162.

Access to the server was explicitly permitted from Administration, Finance, HR and IT. Traffic from other sources was denied.

ACL:

ip access-list extended SERVER_PROTECTION

permit ip 192.168.15.0 0.0.0.31 host 192.168.15.162

permit ip 192.168.15.32 0.0.0.31 host 192.168.15.162

permit ip 192.168.15.64 0.0.0.31 host 192.168.15.162

permit ip 192.168.15.96 0.0.0.31 host 192.168.15.162

deny ip any host 192.168.15.162

permit ip any

Applied this command:

interface Vlan60

ip access-group SERVER_PROTECTION out

This configuration allows the four authorised departmental networks to reach the server while blocking other sources.

SSH Management

SSH version 2 was configured on the access switches to provide secure remote management.

Each access switch was assigned a management IP address on VLAN 70.

List:

- SW1: 192.168.15.194

- SW2: 192.168.15.195

- SW3: 192.168.15.196

Configuration:

ip domain-name network.local

username admin password <REDACTED>

crypto key generate rsa

ip ssh version 2

line vty 0 4

login local

transport input ssh

this command was used:

show ip ssh

Figure 5: SSH Management

Switch Hardening

Unused switch ports were moved to VLAN 999 and administratively disabled. This prevents unused physical interfaces from remaining active and reduces the risk associated with unauthorized device connections.

SSH was used for secure remote management and login banners were configured on the switches.

Configuration:

vlan 999

name UNUSED_PORTS

interface range FastEthernet0/8 - 24 switchport mode access switchport access vlan 999 shutdown

Router Configuration

The router was connected to the Catalyst 3560 through the 192.168.15.224/30 transit network.

Router interface:

interface GigabitEthernet0/0/0

ip address 192.168.15.226 255.255.255.252

no shutdown

Static routes:

ip route 192.168.15.0 255.255.255.224 192.168.15.225

ip route 192.168.15.32 255.255.255.224 192.168.15.225

ip route 192.168.15.64 255.255.255.224 192.168.15.225

ip route 192.168.15.96 255.255.255.224 192.168.15.225

ip route 192.168.15.128 255.255.255.224 192.168.15.225

ip route 192.168.15.160 255.255.255.224 192.168.15.225

ip route 192.168.15.192 255.255.255.224 192.168.15.225

Testing and Verification

Gateway connectivity was tested from end devices in each VLAN.

Administration

ping 192.168.15.1

Result: Successful

Finance

ping 192.168.15.33

Result: Successful

HR

ping 192.168.15.65

Result: Successful

IT

ping 192.168.15.97

Result: Successful

Guest

ping 192.168.15.129

Result: Successful

Server Connectivity Tests

Server connectivity was verified from the authorised departmental VLANs.

- Administration → 192.168.15.162 → Successful

- Finance → 192.168.15.162 → Successful

- HR → 192.168.15.162 → Successful

- IT → 192.168.15.162 → Successful

- Security Verification

Security policies were validated using connectivity tests and ACL hit counters.

Guest Isolation

- Guest → Server → Blocked

- Guest → Internal network → Blocked

Server Protection

- Administration → Server → Allowed

- Finance → Server → Allowed

- HR → Server → Allowed

- IT → Server → Allowed

- Unauthorised sources → Server → Blocked

Figure 6: ACL Security Evidence

Management Connectivity Tests

Management connectivity was verified using VLAN 70.

Tests included:

- ping 192.168.15.193

- ping 192.168.15.194

- ping 192.168.15.195

- ping 192.168.15.196

Successful communication between the core switch and the access-switch management addresses confirmed that VLAN 70 was being carried across the required trunk links.

SSH Verification

SSH connectivity was tested from an Administration PC to each access switch.

Tests:

ssh -l admin 192.168.15.194

ssh -l admin 192.168.15.195

ssh -l admin 192.168.15.196

Results:

- SW1 → Successful

- SW2 → Successful

- SW3 → Successful

Troubleshooting and Resolutions

Issue 1: Native VLAN Mismatch

%CDP-4-NATIVE_VLAN_MISMATCH.

This was due to having different native vlans amongst the switches. Department1 had native vlan was 1, department2 had native vlan 10 and department3 was native vlan 1. I had to change these native vlans to 1 across the switches.

Issue 2: Incorrect Access/Trunk Port Configuration

During troubleshooting, some interfaces intended for end devices had been configured as trunk ports. This prevented normal untagged end-device traffic from being handled as expected.

Resolution:

switchport mode access

switchport access vlan <VLAN-ID>

Verification:

show interfaces <interface> switchport

Issue 3: Trunk Link Down

One uplink interface was initially down because the physical link between the switches was not operational.

Resolution:

- Verified the physical connection

- Enabled the interface

- Verified both sides of the trunk

Verification:

show interfaces

show interfaces trunk

show cdp neighbors

Issue 4: SSH Authentication Failure

SSH connectivity was initially successful at the transport level, but authentication failed.

Resolution:

The local administrative user account was recreated, and the VTY lines were configured to use local authentication.

Configuration:

username admin password <REDACTED>

line vty 0 4

login local

transport input ssh

Result:

Successful SSH login was achieved on SW1, SW2 and SW3.

Configuration Verification Commands

show vlan brief

show interfaces status

show interfaces trunk

show ip interface brief

show ip route

show ip access-lists

show spanning-tree

show cdp neighbors

show ip ssh

show running-config

show mac address-table dynamic

copy running-config startup-config

These commands were used throughout the implementation to verify VLAN membership, interface status, trunking, routing, ACL operation, Spanning Tree state, device discovery, SSH configuration and MAC address learning.

Limitations

The current Packet Tracer topology does not include a connected ISP or external WAN network. The router therefore provides internal routed connectivity to the enterprise network but does not currently provide a validated Internet connection.

NAT/PAT and Internet connectivity were not demonstrated because no external WAN/ISP device was connected to the router in the final topology.

Conclusion

The enterprise network was successfully designed and implemented in Cisco Packet Tracer using VLAN segmentation, Layer 3 switching, inter-VLAN routing, 802.1Q trunking, ACL-based security, SSH management and switch hardening.

Connectivity testing confirmed that the departmental VLANs could communicate with their respective gateways and that authorised departments could reach the server. Guest network isolation and server protection policies were validated using ACL counters and connectivity tests.

The access switches were securely managed using SSH and a dedicated Management VLAN. Unused switch ports were moved to VLAN 999 and administratively disabled.

The project also provided practical experience in troubleshooting native VLAN mismatches, incorrect switchport modes, trunk connectivity, Spanning Tree behaviour and SSH authentication problems.

Overall, the implementation demonstrates practical knowledge of enterprise switching, routing, segmentation, access control and secure network administration.

Appendix A — IP Addressing Summary

Administration

PC1: 192.168.15.2

PC2: 192.168.15.3

Gateway: 192.168.15.1

Finance

PC1: 192.168.15.34

PC2: 192.168.15.35

Gateway: 192.168.15.33

HR

PC1: 192.168.15.66

PC2: 192.168.15.67

Gateway: 192.168.15.65

IT

PC1: 192.168.15.98

PC2: 192.168.15.99

Gateway: 192.168.15.97

Guest

PC1: 192.168.15.130

PC2: 192.168.15.131

Gateway: 192.168.15.129

Server

Server: 192.168.15.162

Gateway: 192.168.15.161

Management

Core: 192.168.15.193

SW1: 192.168.15.194

SW2: 192.168.15.195

SW3: 192.168.15.196

Router Transit

Core: 192.168.15.225

Router: 192.168.15.226