CMPG325 PROJECT

NAME: YOMELELANI DAMANE

STUDENT NUMBER: 51942437

MILESTONE ONE

CLIENT REQUIREMENTS

Client Requirements

1. Client Background

The client for this project is Ditiro Financial Services (Taung), an organisation operating in the Banking and Finance industry. Because the organisation handles financial information and business operations, it requires a network that is reliable, secure, efficient and properly managed.

The network must allow authorised users and departments to communicate, and access required resources while preventing unauthorised access to sensitive internal resources.

2. Client Network Requirements

The client requires a network that meets the following requirements:

2.1 Reliable Network Connectivity

All required departments, end devices and network services must be connected through a properly designed network. Devices must be able to communicate with other devices where communication is permitted.

The network should minimise unnecessary traffic and provide efficient communication between departments.

2.2 IP Addressing

The client has been allocated the network:

192.168.15.0/24

This address range must be used to develop an appropriate IP addressing plan.

The addressing plan should:

- Assign IP addresses logically to the different departments/VLANs.

- Avoid duplicate IP addresses.

- Provide the required number of host addresses.

- Reserve appropriate addresses for network devices and servers.

- Be clearly documented so that devices can be easily identified and managed.

- Allow future expansion where possible.

Subnetting will be used where required to divide the network into appropriate logical networks.

2.3 VLAN Segmentation

The network must use VLANs to separate different groups/departments.

VLAN segmentation is required because it:

- Separates network traffic.

- Improves network organisation.

- Reduces unnecessary broadcast traffic.

- Improves security.

- Allows access restrictions to be applied between different groups.

Each VLAN should represent a logical section of the organisation's network.

2.4 Inter-VLAN Routing

Because devices in different VLANs may need to communicate, the network must support Inter-VLAN Routing.

A Layer 3 switch using Switched Virtual Interfaces (SVIs) will be used to provide routing between the VLANs.

This design allows authorised VLANs to communicate while still maintaining logical separation between departments.

2.5 Guest Network Security

Guest users must be separated from the organisation's internal network.

The guest network must not be allowed to access internal resources. Appropriate access-control measures must therefore be implemented to restrict guest traffic.

Guests should only have access to the services that are intended for them, such as external network/Internet access where applicable.

This requirement is particularly important because Ditiro Financial Services operates in the banking and finance industry and therefore needs to protect internal resources and information.

2.6 Application/File Server

The client has installed a new application/file server.

The server must only be accessible by authorised departments.

The network design must therefore include appropriate security and access-control rules to ensure that:

- Authorised departments can communicate with the server.

- Unauthorised departments cannot access the server.

- Guest users cannot access the server.

- The server has an appropriate and clearly documented IP address.

2.7 Network Security

Security must be considered throughout the network design.

The network should use VLAN segmentation and appropriate access-control mechanisms to prevent unauthorised communication.

The design must ensure that users can access the resources they require for their work without giving unnecessary access to sensitive network resources.

2.8 Network Devices

Appropriate Cisco networking devices must be selected for the proposed topology.

The devices must be correctly configured to provide:

- VLAN connectivity.

- Inter-VLAN routing.

- End-device connectivity.

- Server connectivity.

- Appropriate security controls.

- Reliable network communication.

The selection of devices should be justified according to the requirements of the client rather than simply adding unnecessary equipment.

3. Proposed Network Topology Requirements

The network topology must be logical, organised and scalable.

The proposed topology should clearly show:

- Network switches.

- Layer 3 switching/routing functionality.

- End-user devices.

- Servers.

- VLANs.

- Network connections.

- Appropriate IP addressing.

- Connections between the different network segments.

The topology should make it easy to understand how data moves between departments, the server and other network resources.

A structured topology will also make the network easier to troubleshoot and maintain.

4. Design Decisions and Justification

The main design decisions are based on the client's need for security, reliability, efficient communication and manageability.

VLANs are used to logically separate departments and reduce unnecessary broadcast traffic.

Inter-VLAN routing through an L3 switch/SVIs is used because authorised departments need to communicate with one another while remaining logically separated.

The 192.168.15.0/24 address block is divided into appropriate networks so that IP addresses are organised efficiently and can be easily managed.

Access-control restrictions are required to prevent guest users and unauthorised departments from accessing internal resources.

The application/file server is placed within the internal network and protected through access-control rules, allowing only authorised departments to access it.

These design decisions directly address the client's operational and security requirements.

The final design must satisfy these requirements while providing a network that is secure, efficient, reliable, well-organised and suitable for the client's banking and finance environment.

THE IP ADDRESSING PLAN

The network for Ditiro Financial Services (Taung) uses the address block 192.168.15.0/24 provided by the client. This network contains 256 total IP addresses, ranging from 192.168.15.0 to 192.168.15.255.

To separate departments and improve security, the /24 network is divided into smaller /27 subnets. Each /27 subnet contains 32 addresses, with 30 usable host addresses. The subnet mask used is 255.255.255.224. Each department is assigned its own subnet and VLAN. This makes IP addresses easier to manage, reduces broadcast traffic, and supports security policies between departments.

| VLAN | DEPARTMENTS | NETWORK ADDRESS | DEFAULT GETAWAY | Usable Host Range. | Broadcast Address. |
| --- | --- | --- | --- | --- | --- |
| VLAN10 | ADMINISTRATION | 192.168.15.0/27 | 192.168.15.1. | 192.168.15.2 – 192.168.15.30 | 192.168.15.31 |
| VLAN20 | FINANCE | 192.168.15.32/27 | 192.168.15.33 | 192.168.15.34 – 192.168.15.62 | 192.168.15.63 |
| VLAN30 | HR | 192.168.15.64/27 | 192.168.15.65 | 192.168.15.66 – 192.168.15.94192.168.15.94 | 192.168.15.95 |
| VLAN40 | IT | 192.168.15.96/27 | 192.168.15.97 | 192.168.15.98 – 192.168.15.126 | 192.168.15.127 |
| VLAN50 | GUEST | 192.168.15.128/27 | 192.168.15.129 | 192.168.15.130 – 192.168.15.158 | 192.168.15.159 |
| VLAN60 | SERVER | 192.168.15.160/27 | 192.168.15.161 | 192.168.15.162 – 192.168.15.190 | 192.168.15.191 |
| VLAN70 | MANAGEMENT | 192.168.15.192/27 | 192.168.15.193 | 192.168.15.194 – 192.168.15.222 | 192.168.15.223 |
| RESERVED | FUTURE EXPANSION | 192.168.15.224/27 | N/A | 192.168.15.225 – 192.168.15.254 | 192.168.15.255 |

The first usable IP address in each subnet is assigned to the corresponding SVI on the Cisco 3560 Layer 3 switch. These addresses act as the default gateways for devices in each VLAN. When an Admin PC needs to communicate with a device outside VLAN 10, it sends the traffic to 192.168.15.1, where the Layer 3 switch performs Inter-VLAN routing. The application/file server is placed in its own dedicated Server VLAN 60 to improve security. It uses a static IP address. Using a dedicated subnet makes it easier to apply ACLs. Authorised departments can be allowed to access 192.168.15.162, while unauthorised departments and Guest users can be blocked. VLAN 70 is reserved for network device management, allowing network devices to be identified and managed separately from normal user traffic. The subnet 192.168.15.224/27 is reserved for future growth. It provides up to 30 usable host addresses and can later be assigned to a new department, additional devices, or another network service without changing the existing addressing scheme.

Summary

The IP addressing plan uses 192.168.15.0/24, divided into eight /27 subnets. Seven subnets are allocated to Administration, Finance, HR, IT, Guest, Server, and Management networks, while one subnet is reserved for future expansion. Each VLAN has its own network, subnet mask, and default gateway on the Layer 3 switch. This addressing structure supports VLAN segmentation, Inter-VLAN routing, security controls, easier network management, and future scalability, making it suitable for Ditiro Financial Services' banking and finance environment.

GITHUB

Yomelelani/Ditiro-Financial-Services-Taung-: This repository contains a secure Cisco Packet Tracer network design for Ditiro Financial Services (Taung). It includes subnetting, VLANs, Inter-VLAN routing, ACL security, server protection, IP addressing, and network testing using Cisco devices.